# Statement of Work

# Michigan DNR, 2023 Boundary Surveys

# FRD-Gaylord Unit-FY23-Project#11

- GOVERNMENT REPRESENTATIVE AND SUBMITTAL:  Please review the Statement of Work below, along with the attached maps and respond with a not-to-exceed Quotation for this project by June 12th, 2023.  Also, please note the Delivery Date in Section G below September 1st, 2023 for field work completed and December 29th, 2023 for Maps and LCRC’s. We request this information by close of business on [SEE POSTING]. The Contractor may request a clarification/scoping meeting (or conference call) to better define the project before preparing the submittal. The work on this project must be done according to the specifications of the contract that was previously provided; pertinent paragraphs contained therein should be carefully examined before this proposal is submitted.

- PURPOSE:  The purpose of this survey is to delineate the common boundary between the State of Michigan land and private land for upcoming management activities. This project will be named the “FRD-Gaylord Unit-FY23-Project#11”.

- PROJECT AREA:  This project consists of One (1) Project Areas (shown on the attached maps), located in Cheboygan County, Michigan, more particularly described as:

Project Area: T39N-R03W Section 34, Cheboygan County

CONTROL

Online Remon records show the east line of section 34 and corners around the Private Claim to the NW of section 34 and intersection points with the north and west section lines of section 34.  There are also DNR corner records and a 1961 DNR map of the boundary of the Private Claim.  The Department was also in section 3, T38N, R03W in 2021 with a Preliminary Map.

ACCESS

Access will be challenging with no county roads on section or quarter lines.  Very few seasonal or 2-track trail roads due to low lands.

SURVEY RECORDS

Attached, please find the records that have been made available by the Michigan DNR for this project. An index sheet shows what records of corner records are held in house by the Department that can be obtained if requested along with legal descriptions of State Owned Lands.  It is anticipated that additional research and additional records maybe available from other sources. All survey records research is the contractor’s responsibility.

- ITEMS OF WORK:

- All 1/16th corners use the County Remonumentation Corner Code system

Project Area : T39N-R03W Section 34

Item 1 - Corner Search (2 total):

[Corners don’t have a current LCRC or Monument of Record; may require a monument to be placed along with witnessing, drafting a LCRC, and recordation of said certificate]

Item 2 – Control Corner Measurement (6 total):

[Corners with a LCRC; may or may not require a monument to be placed, may require rewitnessing, may require drafting a LCRC, and may require recordation of said certificate]

Section 34: (6 corners)

Item 3 – Corner Monumentation (Possibly 2 total if no corners found)

Section 34: (2 corners)

Item 3B – Monumentation of Section Subdivisional Corners (7 total):

[Corners don’t have a current LCRC; may require a monument to be placed, along with witnessing, drafting a LCRC, and recordation of said certificate]

Section 34: (7 corner)

Item 9A and 9B – Certificate of Survey/ Plat of Survey - Drawing (1 total showing work done in section 34)

[Map depicting the findings of this survey; vendor may provide a recordable map consisting of multiple sheets OR a non-recordable Large format drawing]

E.	NOT REQUIRED FOR THIS PROJECT Notification letters to adjoining private landowners must be sent prior to commencing field work in the interior of sections.

F.  ENCROACHMENTS:  The extent of any encroachments discovered shall be brought to the attention of the Contracting Officers Representative upon confirmation by the property line survey.

G.	DELIVERY DATE:	The final project submission delivery date: September 1st , 2023 for corners in field and December 29th for Maps and LCRC’s.

Attachments:   	-Project maps

-MDNR Specifications & Exhibits

-State of Michigan Terms and Conditions

Matthew Dontz,  PS 56128

Michigan Department of Natural Resources & Contracting Officers Representative (COR)

5-22-2023