# Statement of Work

# Manistee NF, 2026 Boundary Surveys

# GNA-Manistee-FY26-GNA Hardy Pond

- GOVERNMENT REPRESENTATIVE AND SUBMITTAL:  Please review the Statement of Work below, along with the attached maps and respond with a not-to-exceed Quotation for this project.  Also, please note the Delivery Date in Section G below.  We request this information by noon (12:00 pm) on August 4th, 2026.  Send this to the Contracting Officers Representative (COR): Matthew Dontz by email (preferred .pdf or .docx file) at dontzm@michigan.gov .  The Contractor may request a clarification/scoping meeting (or conference call) to better define the project before preparing the submittal. The work on this project must be done according to the specifications of the contract that was previously provided; pertinent paragraphs contained therein should be carefully examined before this proposal is submitted.

- PURPOSE:  The purpose of this survey is to delineate the common boundary between National Forest and private land for upcoming management activities. This project will be named the “GNA-Manistee-FY26 – GNA Hardy Pond”.

- PROJECT AREAS:  This project consists of Three (3) Project Areas (shown on the attached map), located in Mecosta and Newaygo Counties, Michigan, more particularly described as:

Project Area 1: T13NR10W Sections 17, 18, 19, 20 & 22, Mecosta County, Michigan

CONTROL

A fair percentage of the government corners in these sections have been approved by the County Remonumentation Board.  There are some recorded surveys in sections 17, 18, 19.  Along with County Surveyor records in 17 & 19 and adjacent sections.  Consumers Energy along with Fargo Engineering has done extensive work in many of the sections as well.

ACCESS

Access will be fairly challenging in sections 17 & 22, with not much for county roads or trails.  US 131 will be an obstruction E-W in section 22.  Sections 18-20 will have decent access with a fair amount of county roads and two-track trail roads.

SURVEY RECORDS

Attached, please find the records that have been made available to the Michigan DNR for this project. It is anticipated that additional research and additional records maybe available from other sources. All survey records research is the contractor’s responsibility.  These are most of the records from the USFS office in Cadillac.  Extensive research was completed at the Mecosta County ROD.

- ITEMS OF WORK:

LEGAL DESCRIPTIONS:

T13N, R10W – Project Area 1

Section 17: NE of SE.

Section 18: SE of NE, N ½ of SE, S ½ of SW, SW of SE.

Section 19: S ½ Exc. SW of SW, E ½ of NE, NW of NE.

Section 20: NE of NW & SW of NW.

Section 22: SW of NW & W ½ of NE.

NOTE: Not all deeds are available in Forest Service records. (Most provided in this bid

packet) Missing deeds will need to be

copied from county records. All roads and utilities under permit or easement shall be

tied and shown on survey where they cross section and NFS property lines. All

encroachments on NFS lands shall be tied and shown on survey.

Contact USFS Survey staff to obtain land title and other records relevant to each project listed.

Project Area 1: T13NR10W Sections 17, 18, 19, 20 & 22 – Items of Work will

include;

Item 1 - Control Corner Measurement (25 total) (NOTE: common corners between sections are NOT counted twice)

Section 17: 05

Section 18: 06

Section 19: 05

Section 20: 03

Section 22: 06

Item 2 – Monumentation of Section Subdivisional Corners (10 total): This could easily very for this project if any Remon or existing corners of record are not accepted.

Item 2a – 1/16th corners, 1/64th corners, misc. corners (26 total) (Common corners between sections are NOT counted twice)

Section 17: 03

Section 18: 09

Section 19: 03

Section 20: 05

Section 22: 06

Item 3 and 4 – Property Line Measurement and Property Line Posting (8.25 miles total): - See map.

Section 17: 1.00 miles

Section 18: 2.00 miles

Section 19: 1.50 miles

Section 20: 1.50 miles

Section 22: 2.25 miles

Item 6  –  Certificate of Survey (5 total) 1 per each section.

Item 7 – Land Corner Records – If any Government Corners are established and or

previously recorded L.C.R.C. is outdated by evidence found in field a new L.C.R.C. must

be written and recorded.

Project Area 2: T13NR11W Section 12, Newaygo County, Michigan

CONTROL

75 % of the Government corners for the three sections in this project have been through County Remon board and or a land corner record recorded.  There is a recorded survey in section 12 from Consumers Energy and Norm Ochs.  Along with some Consumer Energy Maps and Fargo maps.

ACCESS

Access in section 12 will be fairly good even though the Hardy Dam Pond runs through the middle of the section.  There are lots of roads and trail roads on both the east and west side of the section.

SURVEY RECORDS

Attached, please find the records that have been made available to the Michigan DNR for this project. It is anticipated that additional research and additional records maybe available from other sources. All survey records research is the contractor’s responsibility.  These are most of the records from the USFS office in Cadillac.  Extensive research was completed at the Newaygo County ROD.

- ITEMS OF WORK:

LEGAL DESCRIPTIONS:

T13N, R11W – Project Area 2

Section 12: SE of NE.

NOTE: Not all deeds are available in Forest Service records. (Most provided in this bid

packet) Missing deeds will need to be

copied from county records. All roads and utilities under permit or easement shall be

tied and shown on survey where they cross section and NFS property lines. All

encroachments on NFS lands shall be tied and shown on survey.

Contact USFS Survey staff to obtain land title and other records relevant to each project listed.

Project Area 2: T13NR11W Section 12 – Items of Work will

include;

Item 1 - Control Corner Measurement (05 total) (NOTE: common corners between sections are NOT counted twice)

Section 12: 05

Item 2 – Monumentation of Section Subdivisional Corners (02 total): This could easily very for this project if any Remon or existing corners of record are not accepted.

Item 2a – 1/16th corners, 1/64th corners, misc. corners (03 total) (Common corners between sections are NOT counted twice)

Section 12: 03

Item 3 and 4 – Property Line Measurement and Property Line Posting (0.75 miles total): - See map.

Section 12: 0.75 miles

Item 6  –  Certificate of Survey (1 total) 1 per each section.

Item 7 – Land Corner Records – If any Government Corners are established and or

previously recorded L.C.R.C. is outdated by evidence found in field a new L.C.R.C. must

be written and recorded.

Project Area 3: T14NR10W Sections 33, Mecosta County, Michigan

CONTROL

All the Government corners and the center have been through the County Remon board.  Not many or no surveys have been recorded.  County Surveyor record to east in section 34.

ACCESS

Access in section 33 will be a little difficult with not much access on the east and south halves of the section.  The entire section is private other than the quarter quarter section being asked to survey.

SURVEY RECORDS

Attached, please find the records that have been made available to the Michigan DNR for this project. It is anticipated that additional research and additional records maybe available from other sources. All survey records research is the contractor’s responsibility.  These are most of the records from the USFS office in Cadillac.  Extensive research was completed at the Mecosta County ROD.

- ITEMS OF WORK:

LEGAL DESCRIPTIONS:

T14N, R10W – Project Area 3

Section 33: NW of SE.

NOTE: Not all deeds are available in Forest Service records. (Most provided in this bid

packet) Missing deeds will need to be

copied from county records. All roads and utilities under permit or easement shall be

tied and shown on survey where they cross section and NFS property lines. All

encroachments on NFS lands shall be tied and shown on survey.

Contact USFS Survey staff to obtain land title and other records relevant to each project listed.

Project Area 3: T14NR10W Section 33 – Items of Work will

include;

Item 1 - Control Corner Measurement (05 total) (NOTE: common corners between sections are NOT counted twice)

Section 33: 05

Item 2 – Monumentation of Section Subdivisional Corners (00 total): This could easily very for this project if any Remon or existing corners of record are not accepted.

Item 2a – 1/16th corners, 1/64th corners, misc. corners (04 total) (Common corners between sections are NOT counted twice)

Section 33: 04

Item 3 and 4 – Property Line Measurement and Property Line Posting (3.25 miles total): - See map.

Section 33: 1.00 miles

Item 6  –  Certificate of Survey (1 total) 1 per each section.

Item 7 – Land Corner Records – If any Government Corners are established and or

previously recorded L.C.R.C. is outdated by evidence found in field a new L.C.R.C. must

be written and recorded.

E.	Notification letters to adjoining private landowners must be sent prior to commencing field work in the interior of sections.

F.  ENCROACHMENTS:  The extent of any encroachments discovered shall be brought to the attention of the Contracting Officers Representative upon confirmation by the property line survey.

G.  DELIVERABLES:  Working control diagram in .dwg and PDF format before staking and line marking begins.  Final SIGNED, SEALED & RECORDED digital copies of Certificate of Surveys and any L.C.R.C.’s.  Hard copies of recorded Certificate of Surveys and L.C.R.C.’s mailed to the COR.

H.	DELIVERY DATE:	The final project submission delivery date: 11/02/2026 for field work (corners and line marking)  12/01/2026 for deliverables (Maps, Certificates, LCRC’s)

Attachments:   	-Project maps

-USFS Specifications

-Exhibits

-State of Michigan Terms and Conditions

Matthew S. Dontz PS 4001056128

Michigan Department of Natural Resources & Contracting Officers Representative (COR)

1732 W. M-32, Gaylord, MI 49735

07-15-2026